ECA-LDNet Evaluation Report

COMPARISON TABLE — RESIDE SOTS Indoor Benchmark

| Method | Year | Attention | PSNR | SSIM | Params |
| --- | --- | --- | --- | --- | --- |
| DCP (He et al.) | 2009 | None | 16.62 | 0.8180 | — |
| DehazeNet | 2016 | None | 21.14 | 0.8470 | 0.009M |
| MSCNN | 2016 | None | 17.57 | 0.8100 | 0.008M |
| AOD-Net | 2017 | None | 19.06 | 0.8500 | 0.002M |
| GFN | 2018 | None | 21.55 | 0.8440 | 0.530M |
| GridDehazeNet | 2019 | Attention | 32.16 | 0.9840 | 0.956M |
| FFA-Net | 2020 | CBAM (Channel+Pixel) | 36.39 | 0.9890 | 4.456M |
| DehazeFormer-S | 2023 | Coordinate Attention | 30.29 | 0.9660 | 1.283M |
| DehazeFormer-B | 2023 | Coordinate Attention | 33.26 | 0.9830 | 25.440M |
| DehazeFormer-L | 2023 | Coordinate Attention | 34.77 | 0.9870 | 25.440M |
| ECA-LDNet (Ours) | 2026 | ECA + PixelAttention | 32.53 | 0.9717 | 1.482M ★ |

ABLATION STUDY SUMMARY — ECA-LDNet Components

| Component | Contribution |
| --- | --- |
| ECA (Channel Attention) | Channel-wise feature recalibration |
| PixelAttention (Spatial) | Spatial focus on hazy regions |
| Physics Correction Layer | ASM-guided refinement (J=I-A/t+A) |
| Depthwise Separable Conv | Param efficiency (1.48M total) |
| VGG Perceptual Loss | Feature-level quality matching |
| Multi-Stage Training | Progressive refinement S1→S2→S3 |
| Charbonnier + Edge + FFT Loss | Robust pixel + frequency learning |

ECA-LDNet RESULTS ON ALL BENCHMARKS

SOTS Indoor  : PSNR=32.5272 dB  SSIM=0.9717

SOTS Outdoor : PSNR=31.9351 dB  SSIM=0.9769

RESIDE-6K    : PSNR=30.2035 dB  SSIM=0.9641

Parameters   : 1,481,871 (1.482M)

LATENCY & SPEED CHECK

Params : 1.482 M

MACs   : 97.116 G

Latency: 81.75 ms / image

Speed  : 12.23 FPS (Fast enough for real-time)

Comparison table saved: ./test_results/csv/comparison_table.csv

Final report saved: ./test_results/final_report.txt

FINAL MODEL SUMMARY

Model       : 1,481,871 params (1.482M)

Latency     : 72.3 ms (13.8 FPS)

RESIDE-6K   : PSNR=30.2035 dB  SSIM=0.9641

SOTS Indoor : PSNR=32.5272 dB  SSIM=0.9717

SOTS Outdoor: PSNR=31.9351 dB  SSIM=0.9769

Results saved to: ./final_model_summary/

ALL TESTS COMPLETE — ECA-LDNet ✓

TESTING COMPLETE ✓